Metrology and Calibration - Author: Eng. Nadia Salib Soliman Salib Egyptian Organization for Standardization & Quality ( EOS) - Contents - Introduction - What is metrology? - Development of measurement - Metrology and Science Advance - Metrology and Technology - Metrology and commerce - Role of Metrology Service 2- Transmission of the unit from the primary standard to working measures and measuring instruments. - Confidence in measurement results - Precision and Accuracy - Concepts of Precision and Accuracy - How to use , improve, and combine uncertainty - Reporting uncertainty as confidence level - International co-operation in the field of metrology - International co-operation in the field of units & standard of measurement (1/18)

Metrology and Calibration ----------------------------------------------------------------- - Introduction - What is metrology? The word “metrology” means the “science of measurement “. Measurement, in its turn, is defined as the quantities determination of properties of bodies or phenomena. Such properties and phenomena may be measured as length, mass, temperature, hardness, electrical resistance or light intensity. The measurement process involves comparison of the “quantity” to be measured with another quantity of the same kind considered as the unit. The result of the measurement process is expressed by an equation of the type: Q = v. U Where Q = quantity measured U = unit of measurement V =number expressing the value found for the quantity Examples of results of measurements are: 5m, 3Kg, 25ºC. - Development of measurements Man used measurements from the most ancient times; even primitive societies used measurements for the barter of goods, for making clothes and for (3/18) Constructing dwelling. Man turned first to parts of his body for use as measures, such as the foot, the arm and the finger. Seeds and stones were used to measure weights. Gradually, complete measurement system were developed and integrated with the system of numbers and arithmetic. Standard weights and measures were created and kept by the highest authority in the land. Scientists studying the great human civilization , whether ancient or modern, always found that they have achieved a remarkable knowledge of measuring techniques and had will organized metrological services . In the age of space science and technology the exploration of space would have been impossible without the highly sophisticated metrological system possessed by the space countries. Metrology received such an important place in the economic structure of societies due to the role it plays in the most domains of life. In particular, metrology is essential in three fields- science, technology and commerce. - Metrology and science advance If we review the advance of science across the ages we find that measurements always played a decisive role in scientific progress. One interesting example of this is the accurate measurements of the speed of falling bodies carried out by Galileo in the beginning of the seventeenth century. (4/18) Galileo showed great ingenuity in using the relatively simple measuring instruments available in his time to the utmost of their precision. In the absence of suitable clock he used a dripping water clock and weighed the water dripped in a given time interval. The measurement of the speed of propagation of light in different directions performed by Michelson with great care paved the way to a new understanding the nature of light and matter, space and time. This new understanding was expressed by Einstein’s Theory of Relativity which modified Newtonian mechanics and, together with them, some of our basic notions of the universe. Similar examples could be multiplied endlessly. They prove that metrology is one of the important bases of science advance. On the other hand the development of science provides mythologists with new phenomena and techniques suitable for used in measurements. Recent development in lasers will probably allow the definition of the unit of length to be based on the wavelength of laser beam whose frequency is fixed with respect to the unit of time, and thus ensure interconnection of two base unit :length and time. - Metrology and technology The achievements of science are used by technologists to improve human life. While scientists discover the laws that govern the universe, technologists apply these laws for practical everyday uses. Measurements are very important tools in the field of technology. (5/18) The industrial revolution was characterized by an enormous growth of productive forces and an intensive application of scientific discovers in engineering. Small scale production was replaced by interchangeable, mass production. Interchangeability means that a complex product such as a machine or an appliance is divided into number of component parts, and each part is produced separately in large quantities in a specialized department. The component parts are assembled after production to form the final product without individual fitting or rework. Interchangeability requires the standardization of types and size, and puts sever conditions on the accuracy of component parts, whose dimensions must be true within a small range of permissible variation or “tolerance”. The limits of permissible variation of dimensions are so chosen as to ensure proper functioning of the part after assembly in the machine. The production of such precision parts presupposes the use of precision machine tools and accurate measuring instruments. The assurance of quality of production is based on the use of measurements performed on the raw materials, during production and on the final product. Such measurements may be of the strength, chemical composition or physical properties. Specifications may specify a minimum limit of quality, an upper limit of undesirable properties or a range of variation of the property. The results of measurement are treated statistically and action leading to quality control is taken. (6/18) In many industrial processes measuring instruments are used for automatic control of important process parameters such as temperature, pressure, etc. Such instruments form part of the loop of automatic regulation and must conform to special conditions regarding their dynamic behavior. It is the task of industrial metrology to ensure the accuracy of the huge number of different instruments used in industrial processes. - Metrology and commerce Measurement equipment used in modern commercial transaction cover a very wide range. They start from counter scales and simple linear and capacity measures, and extend to platform balances and heavy weigh bridges uses for trucks. They also include water and gas meters for domestic and for industrial purposes, meters for petroleum and for milk and wine, fixed tanks and tank-wagons, taximeters, electricity meters, alcoholmeters, and many others. The control of legal weights and measures usually includes the following steps: - The approval of prototypes presented by manufacturers and importers of legal measures and measuring instruments. - The primary verification of measures after production. - The periodic verification of measures at fixed intervals and surprise verification. (7/18) - Role of metrology service The role of national metrology service is to organize the numerous applications of metrology and provide the necessary scientific basis for such applications. This service must perform the following functions: - Define the legal units of measurement in the country and ensure their unification and establish the necessary standards for their representative and reproduction with the highest possible accuracy. This activity is usually referred to as “scientific metrology”. - Supervise the accuracy of measures and measuring instruments used in commercial transaction and ensure their conformity to legal conditions. - Offer the necessary meteorological services to the industry and also to other sectors such as building construction, surveying, etc. By calibrating their instruments and reference measures, and by performing some highly accurate measurements needed by them. - Offer the necessary metrological support of scientific research institutes and universities. - Promote consciousness of the importance of metrology activities for the national economy and disseminate metrological information by mass media and through educational institutions. - Promote efficient training of the necessary specialists and technicians needed by the national metrology system, including the personal needed by the metrology service itself. (8/18) - Transmission of the unit from the primary standard to working measures and measuring instruments The high accuracy and constancy for primary standards mean that they are used very rarely and with the utmost of care. On the other hand in every country a large number of working measures and measuring instruments are used for everyday measurement in science, technology and commerce. To measure the accuracy of these measures they must be related to the primary standard representing the unit. In order to transfer the value of the unit from the primary standard to working Instruments without exposing it to excessive use. The primary standard is used to adjust a secondary standard used in calibration work. How ever, working instruments cannot be compared to the secondary standard directly since they are too numerous and spread in different spaces. Therefore the secondary standard is used to adjust a number a number of reference standards distributed in suitable geographic sites to serve each region of the country. The reference standard in a particular region is the highest reference representing the unit in that region. It has to be kept under proper conditions to conserve its accuracy between calibrations. Usually it is used only to adjust working standards that are used in calibration work. Thus the transmission of the unit from the primary standard to the working Instruments takes place by means of an elaborate system of standards of different levels. (9/18) Primary standard. Method of comparison. Secondary standard. Method of comparison; Method of comparison. Reference standard; Working standard. Method of comparison; Method of comparison; Method of comparison. Reference standard; Working standard; Working instruments. Method of comparison. Working instruments. (10/18) - Confidence in measurements results 3-1Precision and accuracy Concepts of Precision and Accuracy Accuracy, the closeness of a measured value to the true value, includes the concepts of bias and precision. Strictly speaking, the actual error of a reported value is usually unknowable. However, limits of error ordinary can be inferred, with some risk of being correct, from the precision and reasonable limits for the possible bias of the measurement process. A measurement process should be sufficiently precise to minimize the number of replicate measurements required for the intended use. A very precise system may need only a few measurements, even one , to provide data that would not be significantly improved by further replication. Also, a measurement system must be sufficiently precise to identify whether or not biases of a comparable magnitude are present in the system. Precision may be evaluated by the redundant process of replicate measurement. Results on a single material may be used for this purpose, or the information obtained on a number of samples may be pooled. Though precise measurements can serve useful purposes when limited comparisons are required. Accuracy is more often an essential requirement. (11/18) Whenever the true value of the measured quantity is needed , or when data from different laboratories , different methodologies, or that from the same laboratory using the same method over a period of time needs to be interrelated, bias can be a serious problem. Accuracy of a result versus its Precision (or reproducibility): Random Errors Do not affect the accuracy of the result but are the cause of Poor precision. Systematic Errors Do not contribute to loss of precision but are the cause of poor accuracy. Absolute Uncertainty The margin of uncertainty associated with a measurement. (12/18) Uncertainty Uncertainty has traditionally been defined as a range of values, usually centered on the measured value, that contains the true value with stated probability. Relative Uncertainty The absolute uncertainty divided by its associated measurement. Confidence Interval When uncertainty is defined as above , the confidence interval is the range of values that corresponds to the stated uncertainty. Confidence Level Confidence level is the probability associated with a confidence interval. The true value can be expected to lie within ± x units of the measured value with 99% confidence . Error, uncertainty and offset (13/18) The term uncertainty is used widely in describing the results of measurement and denotes an estimate of the bounds of inaccuracy. When a result is measured using a laboratory process, it is important to know how much confidence to have in that value. This requires knowing the number of significant figures in that result and knowing how certain the least significant digit is. To accomplish this, a measurement has to be repeated a number of times , with the result becoming more certain the more measurement are made. The values for all measurement are then averaged to produced the mean. We must also calculate the Standard deviation, an indication of how widely the data is distributed on either side of the mean. In cases where a measurement is repeated many times , about 95% of all measurements fall within plus or minus (±) 2s from the mean. How to use , improve , and combine uncertainties: - Simple Measurements - Using instruments to better than specified uncertainty - Making indirect measurements using two standards - Scaling using several standards Reporting Uncertainty as confidence intervals A confidence interval is the range of measured values that has a certain Probability (depending on the confidence level) of including the true value. (14/18) The confidence intervals are x±sn Where x the average value s the standard deviation n number of measurements 4- International cooperation in the field of metrology *The need for co-operation and effect The need for participation in the international metrological co- operation is arisen from many important reasons especially economic, social and scientific. There is a real truth today that international co-operation in the field of metrology has contributed to development of economy and makes economic co-operation more effective, above all less expensive. International organizations deal with scientific, industrial and legal metrology. The membership of the international metrological organizations makes possible: Partnership in making regulations, active participation in the international calibrations and comparisons, participation in join development programs, improvement of the national measurement standards, and suggesting and lobbying needs of the polish metrology. (15/18) 5- International cooperation in the field of units and standards of measurement The metric system was established in France by law passed in 1793. Through a number of international exhibitions the metric system become known to other countries. The conference convened in Paris in 1875 and led to the adoption of the “Meter convention”. This treaty was signed by 17 countries and it provided for the establishment of three bodies: - The international Bureau of weights and measures ( BIPM ). - The international Committee of weights and Measures (CIPM). - The General Conference of weights and measures (CGPM). The function of each of these 3 bodies are given below. - The international Bureau of weights and measures(BIPM) The task of the BIPM is to ensure world-wide uniformity of measurements and their traceability to the International System of Units (SI). It does this with the authority of the Convention of the Metre, a diplomatic treaty between fifty-one nations. The functions of the BIPM are : - To establish fundamental standards and scales for the principal physical quantities. - To maintain the international prototypes. - To carry out comparisons between national and international prototypes and standards. - To ensure coordination of corresponding measuring techniques. - The international Committee of weights and measures(CIPM) The CIPM meets annually at the (BIPM, Bureau international des poids et mesures), and discusses reports presented to it by its Consultative Committees. It issues an Annual Report on the administrative and financial position of the BIPM to the governments of the Member States of the Metre Convention. (16/18) It is the supervisory body of the BIPM. Its membership consists of 18 persons of member states of meter convention representing different countries. The CIPM edits the international journal “ Metrologia” that publishes articles on the more important work in scientific metrology. - The General Conference of weights and Measures ( CGPM) It is the supreme authority on all matters of units and standards of measurement. It is composed of delegates from all the member countries of the Meter Convention. The Convention of the Metre is an intergovernmental treaty open to all States having diplomatic relations with France (the depository of the Treaty) and willing to pay their annual contribution to the BIPM. The CGPM approves the necessary resolutions leading to the adoption and updating of the new definitions for units symbols,….etc. It confirms the results of new fundamental metrological determinations. The draft resolutions for the CGPM are usually prepared by the CIPM on the recommendations of the respective consultative committee. - International Cooperation in the field of legal metrology To ensure international unification of regulations regarding legal measures and measuring instruments an international convention was signed in Paris in 1955. Establishing the International Organization for Legal Metrology (OIML). The OIML is engaged in the following activities: - It issues international recommendations specifying the , technical and metrological requirements of legal measures and measuring instruments ; -It offers technical documentation and information services to the member countries on the metrological regulations of other countries; -It prepares model laws and regulations for legal weights and measures to be (17/18) adopted by developing countries; - It prepares model laws and regulations for legal weights and measures to be adopted by developing countries ; - It prepares model lists of equipment for central metrology laboratories ,for Regional verification offices and for verification agents (inspectors of weights And measures). The International Committee of Legal Metrology (CIML) is the supervisory body of the BIML (International Bureau of Legal Metrology). It convenes yearly and is in turn under the supervision of the General Conference of Legal Metrology (CGML) which it should convene at least once every 6 years. - Keyword Metrology Measurement SI International System of units Primary standard Working standard &instrument Errors Uncertainty International Cooperation - Reference * Notes on General Metrology by Dr. A EL-Tawil * Philosophy of Calibration by Fluke * Different Web sides on the Internet (18/18)